A  SIMPLE ECT IMAGE RECONSTRUCTION PROGRAM IMAGECON.BAS

PROCESS TOMOGRAPHY LTD.

Version 1.0 January 2011

1. INTRODUCTION

The Imagecon.bas program described in this note is intended to show how ECT images can be constructed using relatively simple software. It has been written in a modern form of Basic (Liberty Basic) which runs under the Windows operating system and can be used in either  interpreted or compiled modes. The program can be easily modified and adapted by users to meet their own requirements.

No attempt has been made to optimise the program. Instead, the emphasis has been on making it  as easy to understand as possible by the use of detailed commenting throughout the program code and by the extensive use of subprograms. This makes the main program structure clearly visible and simplifies modifications to the software.

For simplicity, the program reads data files in ASCII format. This means that the standard PTL  binary capacitance data files and sensitivity matrices must first be converted to ASCII format before they can be used in the Imagecon program. This is carried out as follows:

The binary format capacitance data (.bcp) files produced by the standard PTL ECT software can be converted into ASCII data files using the ECT32 software file conversion utility. If a twin-plane data file is to be imaged, the original binary capacitance (.bcp) file must be converted into two separate ASCII data files which are then read by the image reconstruction program. The converted capacitance data files have the extension .anc (ascii normalised capacitances) and details of the conversion utility are given in section 6.

The original (un-normalised) PTL binary format inverse sensitivity matrix files have been converted to normalised Transpose Sensitivity matrices in ASCII format by PTL and included as standard files in the zip folder which  contains the Imagecon software. These new matrices consist of a set of  P x M inverse matrices, where M is the number of unique electrode-pair capacitance measurements per frame of data  (= 28 for an 8-electrode sensor) and P = 1024  = ( 32 x 32) is the number of permittivity pixels. Only inverse sensitivity matrices based on a 32 x 32 pixel grid are supplied.

The ECT images are constructed using the method described in section 3. A permittivity/concentration model can be applied to correct the values of concentration produced  for a 2-phase mixture.

2. SENSITIVITY MATRIX FILE FORMATS

The data files referred to as Sensor Information Files (.sif) within the ECT32 and Flowan software documentation are in fact the inverse (transpose) sensitivity matrix files in binary format. These files are un-normalised (see section 3 for further information about normalisation).

The ASCII file conversion process used by PTL converts each binary .sif inverse sensitivity matrix file into a normalised  inverse sensitivity matrix file in ASCII (text) format.  The file name extension used for these converted files is .asm (ascii sensitivity matrix).

For circular ECT sensors, the circular cross-section is approximated by 812 active pixels in the 1024 pixel grid. The standard PTL binary sensitivity matrices are optimised for image computation speed and only include data for these 812 pixels, so they have dimensions of (M x 812). However, to simplify the image reconstruction process, the new ASCII format inverse sensitivity matrices contain the full set of pixels( ie M x  1024), where the non-active pixels have zero values. They are used together with a (p x p) pixel geometry map ( p=32) which contains zero values for the non-active pixels and values of 1 for the active pixels.

3. ECT PERMITTIVITY IMAGE RECONSTRUCTION

The two basic equations of ECT are known as the forward and inverse equations.

3.1 The forward equation

The forward equation relates the measured capacitances C to the pixel permittivity values K. It can be expresssed in matrix format as follows:

C = S.K.				(3.1)

where:

C is the column vector of measured capacitances containing the M electrode-pair capacitance values

K is the column vector containing the P pixel permittivity values

S is the (MxP) sensor sensitivity matrix. Note that S must be calculated as a separate exercise  using finite element or other methods.

In ECT we normally work with normalised vaues of capacitance and permittivity, where all of the capacitance and pixel permittivities have the value 1 when the sensor is full and the value zero when the sensor is empty.

Before the measured capacitances can be normalised, a set of calibration measurements must be made. This involves measuring the set of sensor capacitances (Ch) with the sensor filled with the higher permittivity material (Kh) and again (Cl) when the sensor contains only the lower permittivity material (Kl).

The measured capacitances Cm at any arbitrary permittivity distribution can then be converted to normalised values Cn using:

Cn = (Cm - Cl)/(Ch - Cl)			(3.2)

The permittivity is also normalised in a similar manner, that is:

Kn = (Kh - Kl)/(Kh - Kl)			(3.3)

The elements of the sensitivity matrix S are normalised by setting all values of C and K to 1 and adjusting the sensitivity coefficients in S by a normalising parameter to ensure that equation 1 is valid when all values of C and K are 1. That is:

C(1) = Sn.K(1)				(3.4)

where C(1) means that all values of the column vector C are 1, K(1) means that all values of K are 1 and Sn contains the normalised sensitivity coefficients.

3.2 The inverse equation

The forward transform relates the electrode-pair capacitances C to the permittivity distribution K inside the sensor. However, in ECT, what we need to calculate is the permittivity distribution K in terms of the set of measured capacitance-pairs C.

This relationship is the inverse of the forward equation and is therefore known as the inverse equation, which can be written as:

K = Q.C					(3.5))

where Q is the inverse of the matrix S.

In most situations, no exact inverse of S exists and so the fundamental problem that has to be solved in ECT is to find the best approximation to the true inverse transform.

The simplest option is to set the inverse transform Q to be the transpose of the Sensitivity matrix S, which is written as ST.  In this case, the inverse equation  can be written as:

K = ST.C					(3.6)

When ECT images are produced using the transpose sensitivity matrix as the inverse transform, the process is known as Linear Back Projection (LBP). Although the choice of the transpose sensitivity matrix may appear arbitrary, there are good physical reasons for choosing this choice.

The same principles must be used to produce a normalised matrix ST to ensure that the normalised permittivity values produced by equations 2 or 3 are 1 when all the normalised capacaitance pair measurements are 1.

The LBP method produces very approximate and blurred permittivity images. A modified version of the transpose sensitivity matrix, suggested by Tikhonov, produces more focussed ECT images. We refer to this transpose matrix as the Tikhonov transform.

4. PERMITTIVITY/CONCENTRATION MODEL

For a 2-phase mixture, the relationship between the absolute solids concentration XA  and the effective relative permittivity KE can be written as:.

KE = K1[1 + N.XA.(K2 - K1)/(K2 + (N-1).K1 – XA.(K2 - K1))]		(4.1)

where XA is the absolute solids concentration, K1 =1 for air, K2 is the permittivity of the solid material, (for which XA = 1).

The N-value is typically between 3 and 10, where 3 is theoretically correct for spherical particles and the larger numbers are valid for spheroids with their long axes  parallel to the electric field. Note that N = 1 corresponds to the series capacitance model, N=3 to the Maxwell model and N = infinity gives the parallel model.

equation 1 can be inverted to express the concentration XA in terms of the measured effective permittivity KE, with the result:

XA = (KE - K1).[K2 + K1.(N-1)] / [ K1.N.(K2 - K1) + (KE - K1).(K2 - K1)]		(4.2)

Note that XA is the absolute (solids) concentration, where 0 < XA < 1 and K2 is the permittivity of the solid material (XA =1).

5. THE IMAGE RECONSTRUCTION PROGRAM IMAGECON

5.1 SOFTWARE INSTALLATION REQUIREMENTS

The image reconstruction program Imagecon.bas is contained within a zip folder (imagecon.zip) together with a set of data and documentation files. These include sets of standard inverse sensitivity matrix files, an ini file which holds input data and a few sample capacitance data files.

A copy of Liberty Basic must be obtained and installed before imagecon.bas can be run. Details of how to obtain a copy of Liberty Basic are given in Appendix 1. Once Liberty Basic has been installed on the PC,  the files in the imagecon.zip folder must be unzipped to a new folder on the PC, which we will assume is called "C:\Imagecon".

The most convenient way to run the software is to ensure that Liberty Basic is the default application for files with a .bas extension. The imagecon.bas program can then be run simply by double-clicking on it. Otherwise, the program must be opened in Liberty Basic using the File > Open menu dialogue.

5.2 HOW TO USE THE SOFTWARE

For simplicity, the Imagecon.bas software uses fixed dimension windows which have been optimised for display on PC screens having a resolution of 1024 x 768 pixels and above. The program will still run on PCs with lower resolution screens, but the windows generated will overlap.

From within the "C:\Imagecon" folder, double-click on the imagecon.bas file. Liberty Basic  will start and display the program code listing shown below.

Figure 5.1. Liberty Basic Code window

Click on the Run option on the menu bar and select Run again from the drop down menu. The program will run and a set of windows will apppear.

Figure 5.2. Data file window

The first sub-window allows the user to select the capacitance data files to be read, the number of measurement planes and the Inverse sensitivity matrix to be used for image reconstruction.

The Inverse transpose sensitivity matrices supplied are standard LBP matrices (eg lbp8-32_t.asm)  and two sets of Tikhonov matrices (eg tik8_32_1t.asm) with Tikhonov factors of 1 and 10,all  for ECT sensors with 6,8 and 12 electrodes.

Among the sample data files are a pair created by dropping a "golf ball" down a twin-plane 8-electrode ECT sensor.  These data files are the default options when the program is run for the first time.

To run the program with the sample golf ball files, set the number of planes to 2 and select the lbp8-32t.asm matrix option. Leave the Plane1 and Plane2 capacitance files set to the default values. The window should now appear as shown below.

Figure 5.3. Data file window (2)

Now click on the Continue button. The next window shown below will be highlighted for data entry.

Figure 5.4. Input Parameters Window

Click the OK button to confirm the default values in the boxes and then click the Plot Images button. Images will start to be displayed in the Plane 1 and Plane 2 permittivity image windows as shown in the next figures.

Figure 5.5. Plane 1 permittivity image

Figure 5.6. Plane 2 permittivity image

At various stages text will appear in the Input Parameters text window shownbelow. The full text dialogue can be viewed using  the scroll bar.

Figure 5.7. Text window

To quit the program. click on the Exit button in the Input parameters window.

Figure 5.8. Input parameters window (2)

6.  CAPACITANCE DATA FILES

The standard PTL binary capacitance data (.bcp) files should be converted using the ECT32v2 software ASCII Output file creation window. Note that the Include File Comments option should be enabled to ensure that the data file is in the correct format for reading by the Imagecon.bas program.

Figure 6.1. ECT32 File conversion window

Once the data files have been converted, they must be copied to the same folder (c:\Imagecon) as the Imagecon.bas file. The new data files can then be selected using the Sel.P1 and Sel.P2 buttons in the File selection window.

7. PROGRAM NOTES

The Imagecon.bas program consists of a main program which calls a number of subprograms. The software has been extensively commented so it should be relatively easy to understand and modify the code.

When converting binary capacitance data files to ascii format, careful consideration should be given to the number of frames in the converted file, as the Imagecon.bas program takes approximately 1 second to plot each frame image. An arbitrary maximum limit of 1000 frames has been coded into the Imagecon software.

In the Input Parameters window, a frameskip box allows only frames at specific defined intervals throughout the data file to be plotted to save time.

Double-click on the Pause button to pause the frame display sequence and click on the Resume button to restart the image sequence display.

If the program hangs, close the background window, then select Run > Kill Basic Programs > Kill All from the Liberty Basic  menu bar.

If required, a compiled version of the program can be obtained using the Liberty Basic compile option by users who purchase the full version of Liberty Basic.

APPENDIX 1

LIBERTY BASIC

A free trial copy of Liberty Basic can be downloaded from the folllowing web site:

http://www.libertybasic.com/

The free trial version is fully functional and will run the  Imagecon.bas program.