SOFTWARE FOR PTL300E ECT SYSTEMS

1. SOFTWARE SUMMARY

The PTL300E ECT system is supplied with a comprehensive set of control and analysis software. The primary software which is used used to control the ECT system and to capture and image data is the ECT32v2 software. However, additional software for use before and after capturing data is also supplied and a brief summary of each software item is given here.

4.1 ECT32v2: The main control and data capture software for use with PTL ECT systems. Full details are given in sections 2 and 5 of the PTL300E Operating Manual and the ECT32v2 Software User Guide.

4.2 Recal. Advanced calibration software suitable for constructing sensor C/K files. Full details in Appendix 8 of the PTL300E Operating Manual.

4.3 IU2000 Supplementary image reconstruction software for producing a range of enhanced ECT images from data files captured using the ECT32v2 software. Full details are given in Appendix 9 of the PTL300E Operating Manual.

4.4 Plot3d: 2 and 3-dimensional plotting software for use with data files generated by the ECT32 software. Full details in Appendix 10 of the PTL300E Operating Manual.

4.5 Makemap: Sensitivity Matrix generation software for circular ECT sensors. Full details in Appendix 11 of the PTL300E Operating Manual.

4.6 MatECT: Exoperimental set of Matlab utilities for ECT. Requires Matlab (not supplied) to be installed on the PC before use. Full details in Appendix 12 of the PTL300E Operating Manual.

4.7 BCPconvert: File conversion software which converts data files captured using ECT32v2 into a range of other file formats. Full details in Appendix 13 of the PTL300E Operating Manual.

4.8 ECT Toolkit. Diagnostic and maintenance software for the PTL300E ECT system. Full details in Section 7 of the PTL300E Operating Manual.

4.9 ECTcon:  This software is no longer needed.

4.10 ECTRemote: Software for exporting live data (streaming) from the control PC to a second PC. Full details in Section 5 (chapter 30) of the PTL300E Operating Manual.

2. SOFTWARE INSTALLATION DETAILS (Revised February 2018)

2.1 NOTE ON WINDOWS 10

Microsoft operating systems after Windows 7 differ radically from previous versions and can be confusing for new users. Fortunately, 3rd party software (“Classic Shell”) is available, which restores the look, feel and functionality of previous windows versions.

We strongly recommend that the Classic Shell software (http://www.classicshell.net/) is installed on all Windows 10 PCs which are required to run our ECT software and the following instructions assume that this has been done.

2.2 SOFTWARE INSTALLATION OVERVIEW

The ECT software is supplied in a single zip file.  Please download and unzip this to install the software on a suitable PC.

The individual programs on the CD must be installed separately as described below.

1. Unzip the software file to a suitable folder on the PC.

2. Open this folder and use Windows or File Explorer to display the Setup files on the CD.

3. Double-click on the first setup file and follow the on-screen instructions to install the software.

4. Repeat step 3 for each setup file as detailed in section 3.

5. Complete the software installation as dscribed in detail in section 4.

Notes:

1. The programs must be installed separately as described in the instructions listed in section 6 for each utility. Note that the ECT32v2  software is protected by a software key which must be obtained by running the Appcode.exe file on the software CD before the program can be used. Note that this code is unique for each PC on which the software is installed.

2. Answer any questions during set-up with a positive response. Some of the programs will try to run once they have been installed, but will fail until the PC has been re-booted. It is necessary to restart the PC after software installation to ensure correct operation of the software.

3. After the programs have been installed, it is necessary to set up the PC networking parameters as described in section 7 before the software can be used on-line to control the CMU..

3. SOFTWARE INSTALLATION DETAILS

3.1  ECT32 SOFTWARE INSTALLATION

1. Locate the file Setup_ECT32v*.exe.

2. Run this file by double clicking on it. The installation wizard will then run.

3. Accept the default folder (C:\ECT32v2) in which the program is to be installed when requested by the wizard.

4. The ECT32 software will be installed and an ECT software program group window containing a number of icon short cuts will be set up. The ECT32v2 software will attempt to run and at this stage, a Registration window containing a user ID code will appear as shown below.

Figure A7.1.1 Software Registration window

5. Fill in a suitable User name and your organisation name and then copy the User ID code which appears and use the Appcode.exe program in the program folder to generate a Product Registration code. Copy this Registration code into the Registration window and click the OK button.

NB. In view of the mix of lower and upper case characters in the product registration code, it is safest to cut and paste this code directly from the return email to the Registration window (using CTRL C to copy and /CTRL V to paste).

6. Once the code has been entered, click OK. The ECT32 software will start and the Configuration window will appear.

7. If the correct code is not entered, the ECT32 software will not start. The registration window will then appear again next time the ECT32 software is run.

3.1.1 Notes on the Security Code

The ECT32v2 software is protected by a security code system in the form of a Product Registration code (which is unique for each PC on which the software is to be used). A software User ID code is generated during the software installation and the Appcode program must be used to obtain the Product Registration code before the software can be run for the first time on each new PC. Alternatively, a Product Registration code can be obtained from PTL as described in the manuals.

If a Product Registration code is not available, the ECT32 program can be used in a restricted Playback only mode. In this case, the Registration window appears each time the ECT32 software is started and the program can be used in Playback only mode by clicking on the Use in playback only mode button. This facility allows unrestricted use of the ECT32 software with either the example data files provided, or with data files generated by users. It therefore allows the use of the software as an ECT teaching aid by engineering and physics students.

3.1.2  The ECT software group window

Following installation of the ECT32v2 software, the ECT software group window will contain the following icon short cuts.

ECT32v2	ECT Toolkit		ECTremote   		Websites

Uninstall	Documentation

Further icons will be added to this window as additional programs are installed.

Figure A7.1.2 Typical ECT program group window (Windows 10).

As well as the programs, copies of the ECT system documentation can also be accessed using the documentation icons in this window.

There is also a link to the PTL web site, which can only be accessed if the PC is connected to a network with internet access.

3.1.3  Windows 10 note.

In some cases, the ECT software group window shortcut may not be created on the windows Desktop. In this case, copy the folder ECT PTL300E from the installation CD directly to the Windows Desktop.  Only do this if the ECT shortcut is missing from the Desktop.

3.2  IU200 IMAGE RECONSTRUCTION SOFTWARE INSTALLATION

1. Locate the file Setup_IU2000v*.exe.

2. Run this file by double clicking on it. The installation wizard will then run.

3. Either accept the default folder (C:\IU2000) or choose a suitable folder in which the program is to be installed when requested by the wizard.

4. The IU2000 software will be installed and an IU2000 icon button, will appear in the ECT software program group window.

This completes the IU2000 software installation.

3.3  RECAL ADVANCED CALIBRATION SOFTWARE INSTALLATION

1. Locate the file Setup_Recalv*.exe.

2. Run this file by double clicking on it. The installation wizard will then run.

3. Either accept the default folder (C:\Recal) or choose a suitable folder in which the program is to be installed when requested by the wizard.

4. The Recal software will be installed and a Recal icon button will appear in the ECT software program group window.

This completes the Recal software installation.

3.4  PLOT3D IMAGE RECONSTRUCTION SOFTWARE INSTALLATION

1. Locate the file Setup_Plot3dv*.exe.

2. Run this file by double clicking on it. The installation wizard will then run.

3. Either accept the default folder (C:\Plot3d) or choose a suitable folder in which the program is to be installed when requested by the wizard.

4. The Plot3d software will be installed and a Plot3d icon button, will appear in the ECT software program group window.

This completes the Plot3d software installation.

3.5  	MAKEMAP SENSITIVITY MAP GENERATION SOFTWARE 	INSTALLATION

1. Locate the file Setup_Makemapv*.exe .

2. Run this file by double clicking on it. The installation wizard will then run.

3. Either accept the default folder (C:\Makemap) or choose a suitable folder in which the program is to be installed when requested by the wizard.

4. The Makemap software will be installed and a Makemap icon button will appear in the ECT software program group window.

This completes the Makemap software installation.

3.6  BCPCONVERT FILE CONVERSION SOFTWARE INSTALLATION

1. Locate the file Setup_Bcpconvertv*.exe

2. Run this file by double clicking on it. The installation wizard will then run.

3. Either accept the default folder (C:\Bcpconvert) or choose a suitable folder in which the program is to be installed when requested by the wizard.

4. The Bcpconvert software will be installed and a Bcpconvert icon button will appear in the ECT software program group window.

This completes the Bcpconvert software installation.

3.7.  MATECT MATLAB UTILITIES SOFTWARE INSTALLATION   *

1. Locate the file Setup_MatECT*.exe.

2. Run this file by double clicking on it. The installation wizard will then run.

3. Either accept the default folder (C:\MatECT) or choose a suitable folder in which the program is to be installed when requested by the wizard.

4. The MatECT software will be installed and a MatECT icon button will appear in the ECT software program group window.

This completes the MatECT software installation.

* Please note that these Matlab utilities are offered "as is". They were written for use under Maltab v 6.0 and not all of them will work with later versions of Matlab. Users are welcome to modify the code as required.

3.8  FLOWAN SOFTWARE INSTALLATION

1. Locate the file Setup_Flowan*.exe.

2. Run this file by double clicking on it. The installation wizard will then run.

3. Either accept the default folder (C:\Flowan) or choose a suitable folder in which the program is to be installed when requested by the wizard.

4. The Flowan software will be installed and a Flowan icon button will appear in the ECT  software program group window.

This completes the Flowan software installation.

Note that the Flowan software is protected by a security code and a procedure similar to that used for the ECT32v2 software must be followed following installation to obtain a software key.

3.9 DATA EXPORT SOFTWARE INSTALLATION (OPTIONAL )

A single file (ECTRemote.exe) must be copied to the Remote PC to allow it to receive the exported capacitance data.

This file should be installed on the Remote PC as follows:

1. On the Control PC, copy the  ECTRemote.exe file in the folder C:\ECT32v2 to a memory stick.

2. Copy the ECTRemote.exe file to a new folder (C:\REM32) on the Remote PC.

3. Create a shortcut to the file ECTRemote.EXE in the rem32 folder and drag this shortcut to the Windows desktop.

4. Re-name the shortcut icon ECTRemote.

The demonstration data export display software is now installed and ready for use.

3.10 ECT SOFTWARE WINDOW AFTER SOFTWARE INSTALLATION

Following the software installation, the ECT Software group window should appear in a similar format to that shown below. Note that a full set of documentation for the programs can be found in the Document files folder.

Figure A7.2  ECT Program group window

3.11 SOFTWARE INITIALISATION

Once all of the software has been installed, the PC must be re-booted by exiting Windows and restarting the PC. This initialises some of the programs which use Matlab DLL files.

4. SETTING UP THE CONTROL PC FOR NETWORKING

Before the ECT32v2, Toolkit and Flowan software can be used to control the ECT system, the Control PC must be set up to allow basic networking via the ethernet port. The following instructions allow the PC to be used to control the ECT system using a crossover ethernet lead, as described in the Quick Start Instructions (chapter 2.2 of the manual).

4.1 METHOD FOR WINDOWS 7/XP

Open the Control panel on the PC and double-click on the Network Connection icon.

Right click on Local area connection, and click on the Properties button.

Select Internet Protocol (TCP/IP) from the menu list and click on the Properties button.

Uncheck Obtain an IP address automatically and instead check Use the following address.

Insert the following address in the upper part of the Internet Properties (TC/IP) Properties

window.

IP address: 					192.168.0.X

Subnet mask:					255.255.255.0

Default gateway:				192.168.0.1

where X is a unique address for the PC.

Each DAM200E unit has a unique network  (IP) address, which is determined by its serial number. This can be found on the identification plate on the rear panel of the DAM200E unit. The PTL convention is to set X to the value 100 + the serial number of the DAM200E unit. So, for example, if the serial number is 038 then X becomes 138.

The DNS server address section of this window can be left blank.

Click on the OK buttons to exit the network setup software.

4.2  METHOD FOR WINDOWS 10

Open the Control panel on the PC and Select (Left click on) the Network and Sharing Centre option.

Select the Change adapter settings option

Right click on  Ethernet and click the Properties button.

Select Internet Protocol (TCP/IP) from the menu list and click on the Properties button.

Uncheck Obtain an IP address automatically and instead check Use the following address.

Insert the following address in the upper part of the Internet Properties (TC/IP) Properties window.

IP address: 					192.168.0.X

Subnet mask:					255.255.255.0

Default gateway:				192.168.0.1

where X is a unique address for the PC.

Each DAM200E unit has a unique network  (IP) address, which is determined by its serial number. This can be found on the identification plate on the rear panel of the DAM200E unit. The PTL convention is to set X to the value 100 + the serial number of the DAM200E unit.

So, for example, if the serial number is 043 then X becomes 143.

The DNS server address section of this window can be left blank.

Click on the OK buttons to exit the network setup software.

4.3  TO SET UP THE EMBEDDED PC NETWORK ADDRESSES

The final step is to set up the EctToolkit software to point to the correct address for the DAM200E unit connected to the Control PC.

Right click on the ECT Toolkit shortcut in the ECT32 program group window  and click on the Properties button. The ECT Toolkit Properties window will appear as shown below:

Figure A7.3. ECT Toolkit Properties window

Click on the Find Target button. This will browse to the target file for the shortcut as shown in the next figure.

Figure A7.4 . Shortcut target file

Right click on the highlighted target file (EctToolkit). The Ect Toolkit Properties window appears as shown in the next figure.

Figure A7.5. EctToolkit Properties Window

Change the last 2 digits in the URL address to match the serial number on the rear panel of the DAM200E unit. In the above example, this has been set to 39.

Click the Apply button and then click on the OK button to exit this window.

Now click on the ECT Toolkit icon in the ECT software group window on the PC desktop.

This should open the ECT Toolkit software, indicating that an ethernet connection has been successfully established between the control PC and the DAM200E unit.

5. USE OF ECT32 SOFTWARE IN OFF-LINE MODE

Use of the ECT32 software on-line with the PTL300E instrument, requires a product activation user code for the software before it can be used as described above.

However, the software can be used to process and view data off-line in Playback mode (eg to view the supplied sample data files), by selecting the Use in playback only mode button in the Registration window. This facility allows unrestricted use of the ECT32 software with either the example data files provided, or with data files generated by users. It therefore allows the use of the ECT32 software as an ECT teaching aid by engineering and physics students.

6. SENSOR INFORMATION FILES (SENSITIVITY MAPS)

A set of generic sensitivity maps (ssm_12_32.sif etc.) have been supplied with the ECT32v2 software in the (default) configure folder.

A further two sets of Tikhonov maps have also been supplied in a sub-folder, which will produce more focussed (although noisier) images. The tn_32-1.sif set of maps are for a Tikhonov factor of 1 (high-gain)  while the tn_32-10.sif maps are for a Tikhonov factor of 10 (lower gain).

Additional sensitivity maps for circular ECT sensors can be generated using the Makemap software supplied.

7. SAMPLE DATA FILES

A set of sample captured data files have also been installed in the C:\ECT32v2\working\examples\data files folder. These files can be used to test and evaluate any of the above software. Further details are given in the ECT32v2 Software User Guide.

8. THE NEXT STEPS

1. Please read Volume 1 of the User Manual, which should give you a good understanding of ECT technology.

2. Once you have done this, try using the ECT32 software to view some of the example data as described in the ECT32v2 Software User Guide.

3. If you need further information about previous work and applications of ECT, please refer to the list of references and papers on the PTL Web site at: www.tomography.com or contact us by email at enquiries@tomography.com.

In case of any problems, please email us at enquiries17@tomography.com.

Process Tomography Ltd. February 2018